# Computer-Aided Design and Optimization of a Microwave Impedance-Matching Network and Frequency-Selective Filter

# 1. Aim

To analyze microwave impedance matching using a series compensation capacitor and a quarter-wave transformer, and to study a third-order Butterworth frequency response over 8–12 GHz using MATLAB.

# 2. Software Required

MATLAB.

# 3. Procedure

Open MATLAB and create a script named ME_exp9.m.

Clear the Command Window, workspace, and previously opened figures.

Set the design frequency to 10 GHz, reference impedance to 50 ohms, and load impedance to 75 + j25 ohms.

Calculate the series compensation capacitance to counteract the load's positive reactance at the design frequency.

Calculate the quarter-wave transformer impedance and input impedance over the 8–12 GHz frequency sweep.

Calculate the reflection coefficient, S11, return loss, and VSWR for the matched network and the unmatched load.

Calculate the third-order Butterworth filter response and convert it to decibels.

Display the design values at the frequency point nearest to 10 GHz in the Command Window.

Plot S11 comparison, return loss and VSWR, Butterworth filter response, and the real and imaginary parts of input impedance.

Run the script and observe the numerical output and four figures.

# 4. MATLAB Program

clc;
clear;
close all;

% Microwave Impedance Matching and Frequency-Selective Filter

%% 1. Input parameters
f0 = 10e9;                    % Design frequency: 10 GHz
Z0 = 50;                      % Reference impedance: 50 ohms
ZL = 75 + 1j*25;              % Load impedance: 75+j25 ohms
fc = 10e9;                    % Filter cutoff: 10 GHz
fstart = 8e9;                 % Sweep start: 8 GHz
fstop = 12e9;                 % Sweep stop: 12 GHz

f = linspace(fstart,fstop,1001);
w = 2*pi*f;
w0 = 2*pi*f0;

%% 2. Design the impedance matching network
% Cancel the load's positive imaginary part using a series capacitor.
Rload = real(ZL);
Xload = imag(ZL);

Cmatch = 1/(w0*abs(Xload));
Zcomp = -1j./(w*Cmatch);
ZloadComp = ZL + Zcomp;

% Quarter-wave transformer
Zt = sqrt(Z0*Rload);
betaL = (pi/2)*(f/f0);

Zin = Zt.*(ZloadComp + 1j*Zt.*tan(betaL)) ./ ...
      (Zt + 1j*ZloadComp.*tan(betaL));

%% 3. Calculate reflection coefficient
GammaMatch = (Zin-Z0)./(Zin+Z0);
S11Match = 20*log10(max(abs(GammaMatch),1e-12));

% Unmatched load reflection
GammaLoad = (ZL-Z0)/(ZL+Z0);
S11Load = 20*log10(abs(GammaLoad));

%% 4. Calculate return loss and VSWR
ReturnLoss = -S11Match;
VSWR = (1+abs(GammaMatch))./max(1-abs(GammaMatch),1e-12);

%% 5. Frequency-selective Butterworth filter
filterOrder = 3;

H = 1./sqrt(1+(f/fc).^(2*filterOrder));
Filter_dB = 20*log10(max(H,1e-12));

%% 6. Display results
[~,idx] = min(abs(f-f0));

fprintf('MICROWAVE NETWORK DESIGN RESULTS\n');
fprintf('Design frequency = %.2f GHz\n',f0/1e9);
fprintf('Reference impedance = %.2f ohms\n',Z0);
fprintf('Load impedance = %.2f + j%.2f ohms\n', ...
    real(ZL),imag(ZL));
fprintf('Compensation capacitance = %.4f pF\n',Cmatch*1e12);
fprintf('Transformer impedance = %.4f ohms\n',Zt);
fprintf('Unmatched S11 = %.2f dB\n',S11Load);
fprintf('Matched S11 = %.2f dB\n',S11Match(idx));
fprintf('Return loss = %.2f dB\n',ReturnLoss(idx));
fprintf('VSWR = %.4f\n',VSWR(idx));

%% 7. Plot S11 comparison
figure;
plot(f/1e9,S11Load*ones(size(f)),'--','LineWidth',1.5);
hold on;
plot(f/1e9,S11Match,'LineWidth',1.5);
grid on;
xlabel('Frequency (GHz)');
ylabel('S11 (dB)');
title('Impedance Matching Performance');
legend('Unmatched Load','Matched Network');

%% 8. Plot return loss and VSWR
figure;

subplot(2,1,1);
plot(f/1e9,ReturnLoss,'LineWidth',1.5);
grid on;
xlabel('Frequency (GHz)');
ylabel('Return Loss (dB)');
title('Return Loss');

subplot(2,1,2);
plot(f/1e9,VSWR,'LineWidth',1.5);
grid on;
xlabel('Frequency (GHz)');
ylabel('VSWR');
title('Voltage Standing Wave Ratio');

%% 9. Plot filter response
figure;
plot(f/1e9,Filter_dB,'LineWidth',1.5);
hold on;
yline(-3.0103,'--','-3 dB');
grid on;
xlabel('Frequency (GHz)');
ylabel('Magnitude (dB)');
title('Third-Order Butterworth Filter Response');
legend('Filter Response','-3 dB Reference');

%% 10. Plot input impedance
figure;

subplot(2,1,1);
plot(f/1e9,real(Zin),'LineWidth',1.5);
grid on;
xlabel('Frequency (GHz)');
ylabel('Real Impedance (ohms)');
title('Input Impedance - Real Part');

subplot(2,1,2);
plot(f/1e9,imag(Zin),'LineWidth',1.5);
grid on;
xlabel('Frequency (GHz)');
ylabel('Imaginary Impedance (ohms)');
title('Input Impedance - Imaginary Part');

# 5. Output

## 5.1 Command Window Output

MICROWAVE NETWORK DESIGN RESULTS
Design frequency = 10.00 GHz
Reference impedance = 50.00 ohms
Load impedance = 75.00 + j25.00 ohms
Compensation capacitance = 0.6366 pF
Transformer impedance = 61.2372 ohms
Unmatched S11 = -11.14 dB
Matched S11 = -240.00 dB
Return loss = 240.00 dB
VSWR = 1.0000

# 6. Result

The MATLAB model evaluated the impedance-matching network and third-order Butterworth response from 8 GHz to 12 GHz. It calculated the compensation capacitance, quarter-wave transformer impedance, reflection coefficient, return loss, and VSWR at the 10 GHz design frequency, and plotted the frequency-dependent responses.

Note: Results are based on the idealized equations in the supplied script; they are not measured hardware data or a full-wave electromagnetic simulation.