Simulation-Based Electromagnetic Analysis of TEM, TE and TM Propagation Modes in Microwave Transmission Structures

# 1. AIM

To simulate and visualize the electromagnetic propagation modes TEM, TE₁₀, and TM₁₁ using MATLAB and compare their normalized field distributions.

# 2. PROCEDURE

Open MATLAB and create a new script.

Clear the command window, workspace variables, and previously opened figures.

Define normalized x and y coordinate vectors from 0 to 1 using 100 sample points.

Use meshgrid to create a two-dimensional coordinate grid.

Represent the TEM mode using a constant field distribution.

Represent the TE₁₀ mode using the function sin(πx).

Represent the TM₁₁ mode using the function sin(πx)sin(πy).

Create one figure and divide it into three side-by-side subplots.

Display each mode using imagesc, add axis labels and titles, and include color bars.

Run the script and compare the three plotted field distributions.

# 3. MATLAB PROGRAM

clc; clear; close all;

x = linspace(0,1,100);

y = linspace(0,1,100);

[X,Y] = meshgrid(x,y);

TEM = ones(size(X));

TE = sin(pi*X);

TM = sin(pi*X).*sin(pi*Y);

figure;

subplot(1,3,1)

imagesc(x,y,TEM);

title('TEM Mode');

xlabel('x'); ylabel('y');

colorbar;

subplot(1,3,2)

imagesc(x,y,TE);

title('TE_{10} Mode');

xlabel('x'); ylabel('y');

colorbar;

subplot(1,3,3)

imagesc(x,y,TM);

title('TM_{11} Mode');

xlabel('x'); ylabel('y');

colorbar;

sgtitle('Electromagnetic Propagation Modes');

# 4. OUTPUT

# 5. RESULT

The MATLAB program was used to visualize the normalized field distributions of the TEM, TE₁₀, and TM₁₁ modes. The TEM distribution remains constant, the TE₁₀ distribution varies with x, and the TM₁₁ distribution varies with both x and y. Thus, the three mode patterns were generated and compared successfully.

Note: These are illustrative normalized mode distributions defined by the supplied script, not a full-wave electromagnetic field solution for a specified physical waveguide.